```mermaid

stateDiagram-v2

[*] --> LoadConfig

LoadConfig: Загрузка config_t()\nmain.py:174\nprint: Current config: {...}\nmain.py:175

LoadConfig --> ESP32: MicroPython\nmain.py:176

LoadConfig --> Desktop: CPython\nmain.py:239

state ESP32 {

[*] --> StartupLed

StartupLed: Начальная LED-индикация\nmain.py:177

StartupLed --> ResetButtonCheck

ResetButtonCheck: Проверка rst_pad\nmain.py:178

ResetButtonCheck --> ResetConfig: удержание 5 с\nmain.py:179–186

ResetConfig: Сброс config.json\nmain.py:186

ResetConfig --> [*]: machine.reset()

ResetButtonCheck --> SelectNetwork: обычный запуск

SelectNetwork: LTE отключён\nmodem_init = False\nmain.py:190

SelectNetwork --> WiFiConnect

WiFiConnect: wlan_est(False)\nmain.py:191

WiFiConnect --> WaitWiFi

WaitWiFi: Инициализация и ожидание Wi-Fi\nmain.py:143–166\nprint: waiting for network (WIFI)...\nmain.py:151\nprint: ...1\nmain.py:156

WaitWiFi --> WiFiReady: подключено

WaitWiFi --> WiFiFailed: таймаут

WiFiReady: print: waiting for network (WIFI)...2\nmain.py:159\nNTP sync\nprint: ...3\nmain.py:162\nошибка: Could not get update from NTP server\nmain.py:164

WiFiReady --> StartWebThread

WiFiFailed --> RetryWiFi: STA-режим\nmain.py:193

RetryWiFi: Повтор wlan_est(False)\nmain.py:195

RetryWiFi --> WaitWiFi

WiFiFailed --> APSetup: AP-режим\nmain.py:197

APSetup: LED AP-режима\nmain.py:198–199

APSetup --> APWebServer

APWebServer: Web-интерфейс, порт 80\nmain.py:200

APWebServer --> [*]

StartWebThread: print: IP config: (...)\nmain.py:203\nWeb-интерфейс в потоке\nmain.py:204

StartWebThread --> CreateDevice

CreateDevice: uart_device\nmain.py:221–224

CreateDevice --> ConfigureBLE

ConfigureBLE: BLE при отсутствии Wi-Fi\nmain.py:225–226

ConfigureBLE --> ConnectHub

ConnectHub: Создание и подключение HUB\nmain.py:228–231\nDEBUG-логи включены\nmain.py:173

ConnectHub --> HubWork

HubWork: Обработка hub.step()\nmain.py:235–236

HubWork --> HubWork: соединение активно

HubWork --> Reboot: соединение закрыто

Reboot: Перезагрузка ESP32\nmain.py:238

Reboot --> [*]

}

state Desktop {

[*] --> CreateDeviceDesktop

CreateDeviceDesktop: uart_device\nmain.py:244

CreateDeviceDesktop --> ConnectHubDesktop

ConnectHubDesktop: hub.connect()\nmain.py:245–248

ConnectHubDesktop --> StartWebDesktop

StartWebDesktop: Web-интерфейс, порт 3000\nmain.py:250–251

StartWebDesktop --> StartHubLoop

StartHubLoop: Поток hub.step()\nmain.py:253–259

StartHubLoop --> InteractiveShell

InteractiveShell: REPL code.interact()\nmain.py:261

InteractiveShell --> [*]

}

```